Digital Logic Design Laboratory

Lab 3

MSI Combinational Logic (II)

Full name: …………………………………………….

Student number: ………………………………….

Class: ……………………………………………….......

Date: …………………………………………………....

- I. Objectives

In this laboratory, students will study:

-	Understand the operation of combinational logic circuit.

-   The operation of some combinational ICs such as: full adder, decoder, encoder.

- II. Procedure

1. Design the adder with two one-bit binary.

a. Design the half adder two one-bit binary.

Two inputs are A, B. Two outputs are S and C.

Build the truth table and the expressions

| A | B | S | C |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

The simplified expressions:

S=A⊕B

C=AB

Implement the circuit via simulation software and paste the result in here

Make comment on the results

b. Design the full adder two one-bit binary.

Three inputs are Cin, A, B. Two outputs are S and Cout.

Build the truth table and the expressions

| A | B | CIN | S | COUT |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

The simplified expressions:

S:

| A/ BCIN | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 |

→S=A⊕B⊕Cin

Cout:

| A/ BCIN | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 |

→COUT=ACIN+BCIN+AB

Implement the circuit via simulation software and paste the result in here

Make comment on the results

2. 8-to-3 Priority Encoder (Interrupt sorter) – IC 74HC148

a. Investigate IC – 74HC148

Construct the circuit as below:

Figure 1 – Encoder 8-to-3 IC 74LS148

- The outputs are connected to LED displays to determine the logic levels.

- Choose the input data D0 - D7 by switches in the order from SW0 to SW7.

- Control EI by using switch.

- Observe the results and fulfill the truth table of 74HC148.

- What are the functions of   and ?

| Input |  |  |  |  |  |  |  |  | Output |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EI | D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 | GS | A2 | A1 | A0 | E0 |
| 1 | X | X | X | X | X | X | X | X | 1 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0 | X | X | X | X | X | X | X | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | X | X | X | X | X | X | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 0 | X | X | X | X | X | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 0 | X | X | X | X | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 0 | X | X | X | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 0 | X | X | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 0 | X | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |

Implement the circuit via simulation software and paste the result in here

b. Priority encoder

Let’s EI equal to 0, fill the outputs A2, A1, A0 in the following cases

|  | A2 | A1 | A0 |
| --- | --- | --- | --- |
| # Case 1: I3 = I2 = I1 = 0 I7 = I6 = I5 = I4 = I0 = 1. | 1 | 0 | 0 |
| # Case 2: I7 = I2 = 0. I6 = I5 = I4 = I3 = I1= I0 =1 | 0 | 0 | 0 |
| Case 3: All 8 inputs are equal to 0. | 0 | 0 | 0 |

Case 1:

Implement the circuit via simulation software and paste the result in here

Make comment on results

__________________________________________________________________________________________________________________________________________

Case 2:

Implement the circuit via simulation software and paste the result in here

Make comment on results

__________________________________________________________________________________________________________________________________________

Case 3:

Implement the circuit via simulation software and paste the result in here

Make comment on results

__________________________________________________________________________________________________________________________________________

3. 2-to-4 Decoder - IC74HC139

Construct the circuit as below:

Figure 2 – Decoder 2-line-to-4-line IC 74HC139

- 4 outputs (Y0-Y3) are connected to LED display (Led 1-4).

- The data inputs (A, B) and control input (G) are connected to switches.

- Change the states of inputs to fulfill the truth table of IC 74HC139.

| Inputs |  |  | Outputs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Control | Data |  |  |  |  |  |
| G | B | A | Y0 | Y1 | Y2 | Y3 |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | X | X | 1 | 1 | 1 | 1 |

Implement the circuit via simulation software and paste the result in here

B1riefly describe the operation of the IC

4. 3-TO-8 Decoder– IC 74HC138

Construct the circuit as below:

Figure 3 – 3-to-8 Decoder/demultiplexer  - IC 74HC138

- 8 outputs are connected by using LEDs.

- The inputs are controlled by switches.

- Observe the results and fulfill the truth table

| INPUT |  |  |  |  |  | OUTPUT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E3 | E2 | E1 | C | B | A | Y0 | Y1 | Y2 | Y 3 | Y4 | Y 5 | Y 6 | Y7 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0 | X | X | X | X | X | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| X | 1 | X | X | X | X | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| X | X | 1 | X | X | X | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Implement the circuit via simulation software and paste the result in here

Briefly describe the operation of the IC

5. Design combinational circuits using decoders and OR gate

- Implement Boolean expression using IC 74HC138 & OR gate.

- The data inputs A, B, C are connected to switches.

- The control inputs are in suitable levels.

- Implement the circuit and verify the truth table

a. f=x'yz'+xz

Establish the truth table

| X | Y | Z | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

Implement the circuit via simulation software and paste the result in here

Verify the truth table and make comment on the results

b. f=x'yz+x+y'z'

Establish the truth table

| X | Y | Z | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

Implement the circuit via simulation software and paste the result in here

Verify the truth table and make comment on the results